# NATIONAL

# SENIOR CERTIFICATE

# GRADE 10

# TECHNICAL SCIENCES

# MEMORANDUM

# JUNE 2017

MARKS		:  100

This memorandum consists of 4 pages

QUESTION 1

| 1.1 | A | B | C | D |
| --- | --- | --- | --- | --- |
| 1.2 | A | B | C | D |
| 1.3 | A | B | C | D |
| 1.4 | A | B | C | D |
| 1.5 | A | B | C | D |
| 1.6 | A | B | C | D |
| 1.7 | A | B | C | D |

[14]

QUESTION 2

2.1

    - Mechanical advantage (MA) is the ratio of load to effort 		  (2)

    - Normal force is the perpendicular force exerted by a surface

on an object that touches the surface. 		          (2)

  - MA= LOADEFFORT

= 700350

= 2										               (3)

    - 3m										   (1)

    - To find reaction at Q take moments about point P:

MCW     = MACW

(100x 2)  = Q x 5

200 = 5 Q												Q= 40 N									              (4)

    - To find reaction at P take moments about point Q:

MCW     = MACW

(P x 5) = (100 x 3)

5 P = 100 x 3

P = 60 N			(4)

    - Class 1 lever

Load 		Effort 		Fulcrum 			(6)

[22]

QUESTION 3

- A substance that consists of one type of atoms or particlesthat has

chemicaland physical properties that does not change.

OR

A substance that has single type of material (elements or compounds)	                         (2)

    - Mercury , gold, Iron, lead, potassium				(5)

    - Carbon, oxygen , chlorine, fluorine 				            (4)

    - Leadsupplied, Copper sulphide					            (2)

    - Lead chloride 								(2)

    - Potassium oxide 							            (2)

    - Copper chloride 								(2)

3.3 Halogens										(2)

3.4 Cation is a positively charged atom or molecule.

Anion is a negatively charged atom or molecule.				            (4)

3.5

    - NaBr									(2)

    - CaO								            (2)

    - HCℓ								            (2)

[31]

QUESTION 4

    - 10kg = 10kg x 1000 = 1000g					             (3)

    - C = (5 ÷ 9) x (F - 32) = (5 ÷ 9) x (98 - 32) = 36.670C		             (3)

4.1.3	400 000 000 m.s-1 = 4, 0 x 108m.s-1				                   (2)

4.1.4	0.5 hours =(0.5 x 60 x 60) = 1800 s					 (2)

- speed = dt= 20020 = 10 m.s-1							             (4)

    - Fres. = F1 + F2

(20 N + 75 N) = 95 N (on the positive direction)				(2)

    - Fres. = F1 + F2

(20 N – 75 N) = - 55 N(on the negative direction)			            (2)

- a =Vf-Vit

a =2.5-15.55 = -2.6 m.s-2							            (5)

[23]

Question 5

- Gravitational potential energy is energy an object has because of its

position from the surface of the earth.

Kineticenergyis the energy of an object due to its motion			            (4)

- Ep= mgh

=4 x 9.8 x 2

= 78.4 J 										(3)

- EK= 12  mv2

54 =12(m) (3)2

m =12 kg										(3)

[10]

TOTAL : 100